# Annex 2 to the Tender Documentation

# Technical Specification of the Subject-Matter of the Public Contract

# Public Contract and Tenderer Identification Details

| Public contract title: | Acquisition of the technology for production innovation |
| --- | --- |
| Type of tender: | public contract not regulated by Act No. 134/2016 Coll., on Public Procurement (hereinafter referred to as “PPA”) |
| Type of public contract: | deliveries |

| Tenderer’s name: | Click here and enter the text. |
| --- | --- |
| Tenderer’s registered office: | Click here and enter the text. |
| Contact details: | Click here and enter the text. |
| ID Number (IČO): | Click here and enter the text. |

This document may only be signed by a person authorised to represent the Tenderer in the Tender.

# Technical Specification of the Subject-Matter of the Public Contract

This document sets out the minimum required technical parameters of the subject-matter of the public contract – a new production technology – an injection moulding machine for the production of wax models (hereinafter referred to as the “Subject-Matter of the Public Contract” or the “Equipment”).

In case there is any ambiguity regarding compliance with a parameter, the Contracting Authority may require the Tenderer to submit product datasheets issued by the manufacturer of the offered equipment or samples or models of the equipment to clarify the bid within the meaning of Section 46 (1) of the PPA.

The capacity, quality and technical parameters and requirements for the subject-matter of the public contract defined by the Contracting Authority as well as the values specified for these parameters are set as the minimum permissible. Therefore, the Tenderers may offer equipment with better parameters and properties in the functionalities required by the Contracting Authority.

# Equipment offered by the Tenderer

For each listed item (row) in the table, the Tenderer shall indicate the specific technical parameters of the Equipment offered or, for non-quantifiable requirements, it shall indicate YES/NO, i.e. whether or not the Equipment meets the requirement. Furthermore, the Tenderer will specify the manufacturer and type designation of the Equipment offered.

In order for the bid to be assessed and evaluated, the Tenderer must meet all technical parameters of the Equipment required by the Contracting Authority.

Requirements for an injection moulding machine for the production of wax models:

| Minimum technical requirements required by the Contracting Authority: |  | Technical parameters offered by the Tenderer, please specify YES/NO – based on the information below: |
| --- | --- | --- |
| General characteristics: |  |  |
| Injection moulding machine for the production of wax models, C-type frame, vertical closing, min. clamping force 80 t, bottom injection, lower sliding table. Dual tanks for processing two different waxes. |  | Click and select YES/NO. |
| Description: |  |  |
| hydraulic injection moulding machine for the production of wax models, C-frame |  | Click and select YES/NO. |
| vertical mould clamping |  | Click and select YES/NO. |
| table opening | 750–820 mm | Click and specify the parameter value |
| minimum table clamping | 100 mm | Click and specify the parameter value |
| movable top plate |  | Click and select YES/NO. |
| bottom injection in the centre of the bottom fixed plate |  | Click and select YES/NO. |
| sliding table, equipped with M10 threads, layout according to the supplied drawing (drawing attached) |  | Click and select YES/NO. |
| variable machine speed during the working phase – two speeds. |  | Click and select YES/NO. |
| min. clamping force of the plates | 80 tons | Click and specify the parameter value |
| size of clamping plates min. | (width) 800 x (depth) 1000 mm | Click and specify the parameter value |
| temperature control of the wax injected into the mould |  | Click and select YES/NO. |
| regulation of the wax flow depending on the desired mould filling time |  | Click and select YES/NO. |
| working with two types of waxes – two independent mixers |  | Click and select YES/NO. |
| easy switching between wax types |  | Click and select YES/NO. |
| CE certificate |  | Click and select YES/NO. |
| Machine control: |  |  |
| control system and display unit |  | Click and select YES/NO. |
| easy and quick program change |  | Click and select YES/NO. |
| unlimited program capacity |  | Click and select YES/NO. |
| control of all adjustable machine parameters including heating and cooling of the machine plates |  | Click and select YES/NO. |
| remote desktop access for machine maintenance |  | Click and select YES/NO. |
| access security - user rights |  | Click and select YES/NO. |
| digital readout of the pressure on the plate |  | Click and select YES/NO. |
| digital control and monitoring of the plate position |  | Click and select YES/NO. |
| real-time graphical representation of the current wax temperature at the nozzle tip for each injection |  | Click and select YES/NO. |
| Control panel on the movable arm |  | Click and select YES/NO. |
| safety to be guaranteed by “two-hand operation” |  | Click and select YES/NO. |
| Injection system: |  |  |
| programmable wax flow control |  | Click and select YES/NO. |
| programmable pressure regulation |  | Click and select YES/NO. |
| complete real-time control of the injection moulding process with graphical representation |  | Click and select YES/NO. |
| Required units of regulated variables: temperature in degrees Celsius injection pressure in bar press pressure in bar wax volume flow rate in cm3/s injected volume in cm3 |  | Click and select YES/NO. |
| Nozzle, its position, mould centering, injection pressure, size: |  |  |
| flat nozzle, injection diameter 12 mm (drawing attached) |  | Click and select YES/NO. |
| in the centre of the table from below, fixed position |  | Click and select YES/NO. |
| size of centering ring for correct position of the mould dia. 63 mm (drawing attached) |  | Click and select YES/NO. |
| injection pressure | 3.5 to 70 bar | Click and specify the parameter value |
| injection speed | 0 - 0.2 l/sec. | Click and specify the parameter value |
| real-time control of the injection moulding process |  | Click and select YES/NO. |
| wax recirculation system – preventing wax build-up |  | Click and select YES/NO. |
| injection volume | between 15–20 litres of wax | Click and specify the parameter value |
| adjustable automatic flushing of the injection unit |  | Click and select YES/NO. |
| 5-stage injection system with adjustment of flow rate and pressure for each stage, with visualisation of current status and history on a touch screen |  | Click and select YES/NO. |
| Cooling and heating of the machine’s clamping plates: |  |  |
| temperature of each of them must be controlled independently |  | Click and select YES/NO. |
| temperature monitoring using alarms – if the temperature is out of tolerance |  | Click and select YES/NO. |
| Wax preparation: |  |  |
| two wax units, each of them with a capacity of | min. 110 litres | Click and specify the parameter value |
| matching two-zone wax melter with vacuum wax replenishment system |  | Click and select YES/NO. |
| vacuum pellet loader automatically keeping the melter filled with wax |  | Click and select YES/NO. |
| adequate thermal power for wax melting and cooling |  | Click and select YES/NO. |
| wax melting capacity | 20–25 kg/h | Click and specify the parameter value |
| Requirements for working with wax: |  |  |
| temperature of filled wax 62±3 °C |  | Click and select YES/NO. |
| wax temperature regulation |  | Click and select YES/NO. |
| multi-zone control for precise temperature regulation |  | Click and select YES/NO. |
| ability of the machine to work with wax in a paste-like state |  | Click and select YES/NO. |
| real-time display of the actual wax temperature at the nozzle for each injection |  | Click and select YES/NO. |
| we demand a precise wax temperature coming out of the nozzle within ± 1 °C |  | Click and select YES/NO. |
| Language version of the machine control system: |  |  |
| Czech language |  | Click and select YES/NO. |
| Colour design of the machine: |  |  |
| in colours according to the logo and corporate design of the client – blue (RAL 5017) grey (RAL 7035) |  | Click and select YES/NO. |
| Requirements for the documentation: |  |  |
| in the English language, conforming to current standards in both printed and digital format |  | Click and select YES/NO. |
| Standard requirements for connections: 380 V, 50 Hz, 3 phases, air pressure 5–6 bar |  | Click and select YES/NO. |

| Manufacturer of the equipment | Click or tap here to enter text. |
| --- | --- |
| Type designation of the equipment | Click or tap here to enter text. |

Location, Date (location, date)

Name, position, signature